# Piper PA28-181 Checklist

INTERIOR PREFLIGHT

Required Documents.....……On Board

Master Switch…………...………...Off

Avionics Master……..……...……..Off

Fuel Selector..…………..………Cycle

Pitot/Static Drain…….Drain (2-3 Sec)

Master Switch……….……..………On

Fuel Quantity Gauges..…………Check

Flaps…………………………...Extend

Aircraft Lights………….…..On/Check

Pitot Heat…………………...On/Check

Stall Warning Horn………….....Check

All Electrical Switches………...….Off

EXTERIOR PREFLIGHT

RIGHT WING:

Flap…………………..………...Check

Aileron………………..……..…Check

Nav/Anti-Collision…….……….Check

Leading Edge…………………..Check

Fuel Vent……………………….Check

Fuel Tank Sump………………...Drain

Fuel Quantity…………………..Check

Main Wheel/Strut	………………Check

Brakes………………………….Check

Air Inlets…………………….…..Clear

NOSE:

Windshield……………...……...Check

Oil………………………….6 Qts Min

Cowling……………………...…Check

Alternator Belt…...……………..Check

Propeller…………………...…...Check

Nose Wheel/Strut	………………Check

Air Inlet………………...………Check

Fuel System Sump…………........Drain

LEFT WING:

Air Inlet	…….….………..……..Check

Brakes……………...……..….…Check Main Wheel/Strut	……….……...Check

Fuel Tank Sump………..……….Drain

Fuel Quantity……….…….....….Check

Fuel Vent……………….………Check Leading Edge…………….…….Check

Pitot Head…………………..….……Check

EXTERIOR PREFLIGHT CON’T

Nav/Anti Collision.…….…………...Check

Aileron.…………………..………….Check

Flap	.……………………..…………..Check

FUSELAGE/TAIL:

Antennas.………………..……….….Check

Lights.……………………..………...Check

Stabilator.………………….………..Check

Rudder.…………………….………..Check

Baggage Door.……………..………..Check

#### BEFORE START

Preflight	.………………….……..Complete

Passenger Briefing ……………...Complete

Belts/Harnesses/Seats.…….………..Secure

Circuit Breakers.………….……………..In

Avionics Master.………….…………...Off

Fuel Selector.………………....Fullest Tank

ENGINE START

Throttle.……………….……1/4 Inch Cold

½” Hot

Mixture.……………….……...............Rich

Primer ……………………………4-6 Cold

3-4 Hot

Master Switch..……….………………...On

Aircraft Lights.……….………………...Set

Fuel Pump..…………….……………….On

Brakes…………………….…………..Hold

Propeller Area..……………….…...…Clear

Starter..…………………………….Engage

Throttle..………………….800-1,000 RPM

Oil Pressure..………………………..Check

BEFORE TAXI

Avionics Master.…………….…………On

Ammeter………….………..Check Positive

Flaps………………………………….....Up

Mixture.…………………….……..….Lean

Fuel Pump.………………….………….Off

Fuel Selector………………...Switch Tanks

Transponder.…………...…..Set Code/ALT

Avionics.……………….…………...….Set

Flight Instruments.…….………..Set/Check

Aircraft Lights..…………….……..……Set

Nav Lights………………….….……….On

Brakes.………………….…………….Test

RUN-UP

Brakes..……………………….………Hold

Flight Controls.……..……...Free & Correct

Fuel Selector.………...……….Fullest Tank

Circuit Breakers……………………..Check

Mixture.……………..………….……..Rich

Throttle.………………..………2,000 RPM

Magnetos.…………….……………..Check

*Max drop 175 RPM, Max Diff 50 RPM*

Carburetor Heat……….……………Check

Oil Pressure.…………….…………..Check

Fuel Pressure.………….…………....Check

Oil Temp.……………….……….….Check

Annunciator Lights …………………Check

Throttle..……………….……….Idle Check

Throttle.…………….……..800-1000 RPM

Radios/Nav…………….……………….Set

Mixture………………….Re-Lean for Taxi

Magnetos..……………….……………Both

BEFORE TAKEOFF

Trim..……….………..………..Set Takeoff

Flaps..…………………………...……...Set

Mixture...…………….….…………….Rich

Aircraft Lights..………….…………..…Set

Fuel Pump..………….….………………On

Flight Instruments/Avionics……………Set

Transponder..……………………..Set ALT

Cabin Door/Windows…..................Latched

Takeoff Briefing…………..……..Complete

NORMAL TAKEOFF

Heading.…..............Check Correct Runway

Throttle.………………….……...Full Open

Rotate..…….………………… 48-53 KIAS

CLIMB

Flaps..……….……………..…………...Up

Best Rate …………………….Vy 76 KIAS

Cruise Climb …………………….87 KIAS

CRUISE

Throttle.……….…………...……..…….Set

Fuel Pump.……………….…………….Off

Mixture.…………...………………...….Set

Aircraft Lights.………………………... Set

Fuel Selector..... Switch tanks every 30 min

DESCENT

Weather/Avionics .………..Checked

Approach Briefing………..Complete

Descent Power..………….………Set

Carburetor Heat …………………Set

Fuel Pump..……………….……...On

Mixture..…………………...…….Set

Fuel Selector..……….…Fullest Tank

# BEFORE LANDING

Fuel Pump..…………….…..……On

Mixture..………..……....………Rich

Throttle..………………...……….Set

Aircraft Lights..…….....................Set

Flaps...…………….………...…...Set

Approach Airspeed.....…….66 KIAS

AFTER LANDING

Flaps..………………….………...Up

Trim………………….....Set Takeoff

Carburetor Heat …………………Off

Aircraft Lights..…….….………...Set

Pitot Heat………………………..Off

Fuel Pump.…………….………...Off

Mixture..……………………….Lean

SHUT DOWN

Avionics Master.……….……….Off

Throttle.………………….……...Idle

Magneto..……………………..Check

Mixture..……………….Idle Cut-Off

Magnetos..…………….…………Off

All Switches..………….………...Off

Aircraft Lights.………….……….Off Nav Lights……………….……..ON

Master Switches………………...Off

Brakes……………….…….Released

### Important Speeds (KIAS)

VS0…………….......……………...49

VS1……………........……………..55

VR…………............…………..48-53

VX………………….……………....64

VY……………………..……………76

VFE…………........………...........102

VA (2550 lbs.)….……………….113

VNO…………………..................125

VNE...…………..........…………..154

Best Glide…………..………........76

Approach……….......…………….66

Piper PA28-181 Maneuvers

Normal Takeoff

*Before Takeoff Checklist Complete*

Flaps …………………………………... 0˚

Throttle …………………………Full

Engine Instruments ……Verify Green

Rotate ……………………….53 KIAS

Initial climb………………….76 KIAS

Enroute climb………………..87 KIAS

Normal Landing

*Before Landing Checklist Complete*

Entry/Downwind …………….BCGUMPS

Abeam Threshold –

Throttle ………………..1500 rpm

Airspeed ……………….90 KIAS

Flaps ………………………….10°

Base –

Airspeed…..……………80 KIAS

Flaps …………………………25°

Final –

Airspeed……..…………70 KIAS

Flaps ………………………….40°

Touchdown ……Just above stall speed

Short Field Takeoff

*Before Takeoff Checklist Complete*

Flaps ………………………………25°

Runway ……………Use All Available

Brakes……………………………Hold

Throttle……………………………Full

Engine Instruments……..Verify Green

Brakes………………………...Release

Rotate ………………………49 KIAS

Climb Out ……………………..Vx 64

When Clear of Obstacle

Accelerate to …………………..Vy 76

Flaps …………………Retract Slowly

Short Field Landing

*Before Landing Checklist Complete*

Downwind …………………..BCGUMPS

Approach slightly steeper than normal

Abeam Runway Threshold

Airspeed ……………………90  KIAS

Flaps ………………………………10°

Base –

Airspeed …………….............80 KIAS

Flaps ………………………………25°

Final –

Airspeed …………….............70 KIAS

Flaps ………………………………40°

Threshold ……………..66 KIAS

Touchdown ………..Just above stall speed

Use max. foot and aerodynamic braking

Soft Field Takeoff

*Before Takeoff Checklist Complete*

Flaps…………………………………..25°

Flight Controls………………….Full Back

Engine Instruments…......…..Verify Green

Throttle……………………………….Full

Rotate……………………...Min. Airspeed

Ground Effect …………..Remain until Vy

Or Runway Threshold

Climb ……………………………....Vy 76

Flaps ……………………...Retract Slowly

Soft Field Landing

*Before Landing Checklist Complete*

Entry/Downwind……………..BCGUMPS

Abeam Runway Threshold

Airspeed ……………………90  KIAS

Flaps ……………………………....10°

Base –

Airspeed …………….............80 KIAS

Flaps ………………………………25°

Final –

Airspeed ……………............70 KIAS

Flaps ……………………………...40°

Threshold ……………..66 KIAS

Touchdown ………..Just above stall speed

Yoke……………………………...Full Aft

Keep nosewheel off ground as long as possible, holding full backpressure. Use min. wheel braking

Go-Around

Throttle ………………………………Full

Carburetor Heat ……………………...Off

Flight Control ……………. Start Climbout

Flaps ……………………… Retract to 25°

Airspeed ……………….Initially 55 KIAS

then Vy 76

Flaps …….When >70 KIAS, Retract to 0°

Slow Flight

Clearing Turns ……………...….Complete

BCGUMPS……………………..Complete

Throttle………………… 1500-1700 RPM

Flaps……………………40° (in white arc)

Altitude………………………….Maintain

Throttle .................................Add power to

Maintain 50-55 KIAS Bank…………………...…..Shallow Turns

Recovery……...……Go-around procedure

Power Off-Stall

Clearing Turns …………………Complete

BCGUMPS …………………….Complete

Throttle………………… 1500-1700 RPM

Flaps ………………………………….40°

Airspeed ………………………..66 KIAS

Descent ..................... Initiate 500 FPM to

simulate final approach

Throttle……………………………….Idle

Stall…………………….…Pitch to Induce

Recognize and recover from stall

Stall Recovery:

Go-Around Procedure

Minimize altitude loss

Power-On Stall

Clearing Turns …………………Complete

BCGUMPS …………………….Complete

Throttle ………………... 1500-1700 RPM

Flaps……………………………...0° - 25°

Airspeed………………………… 65 KIAS

Throttle………………………...2300 RPM

Stall...............................…..Pitch to Induce

Recognize and recover from stall.

Stall Recovery:

Flight Controls……………….Nose Down

Throttle……………………………….Full

Secondary Stall

Clearing Turns ……………..Complete

BCGUMPS ………………...Complete

Throttle……………………1500 RPM

Flaps ………………………………40°

Airspeed …………………….66 KIAS

Descent ................. Initiate 500 FPM to

simulate final approach

Throttle…………………………...Idle

Stall…………………..Pitch to Induce

Recognize and recover from stall.

Flight Control….Release back pressure

Throttle……………………………Full

Flight Control…Increase back elevator

pressure to initiate as secondary stall

Recognize and recover from stall.

Stall Recovery:

Flight Controls……………Nose Down

Go-Around Procedure

Minimize altitude loss

Steep Turns

Clearing Turns ……………..Complete

BCGUMPS ………………...Complete

Throttle……………….……2300 RPM

Airspeed……………………100 KIAS

Bank………………………45° Private

50˚ Commercial

Roll Out …………….Original heading

Repeat …………….Opposite direction

Elevator Trim Stall

Clearing Turns……………...Complete

BCGUMPS…………………Complete

Throttle……………………1500 RPM

Flaps………………………………40°

Airspeed……………………..75 KIAS

Throttle……………………………Idle

Trim…………………….Full Nose-Up

Throttle…………………………..Full

Stall Recovery:

Flight Controls……………Nose Down

Trim…………………Maintain control

Flaps……When >70 KIAS, retract to 0

XXXXXXXXXX

Piper PA28-181 Maneuvers

Cross-Controlled Stall

Clearing Turns ……………..Complete

BCGUMPS ………………...Complete

Throttle ……………………1500 RPM

Airspeed……………………..70 KIAS

Stabilized Descent…………..Establish

Stall….Pitch to Induce, Cross Controls

Recognize and recover from stall.

Stall Recovery:

Flight Control …Release back pressure

Throttle……………………………Full

Altitude………………...Minimal Loss

Airspeed……...Accelerate to Vx or Vy

before the final flap retraction

Accelerated Stall

Clearing Turns……………...Complete

BCGUMPS…………………Complete

Throttle……………………1500 RPM

Flaps………………………………...0°

Airspeed……………………..80 KIAS

Bank……………………………….45°

Pitch….……Maintain Alt. Induce Stall

Recognize and recover from stall.

Stall Recovery:

Flight Control…Release back pressure

Throttle……………………………Full

Altitude………………...Minimal Loss

Airspeed…………….Accelerate to Vy

SEE PIPER ARCHER STANDARDIZATION MANUAL FOR AMPLIFIED PROCEDURES

Lazy Eights

Altitude……..……Min Alt 1500 AGL

Clearing Turns……………...Complete

BCGUMPS…………………Complete

Throttle……………………2300 RPM

Airspeed…………………….95 KIAS

(Constant change of pitch and roll rate)

45° 	Point…..Max. Pitch UP, Bank 15°

90°	Point……..Level. Pitch, Bank 30°

135° Point…..… Max. Pitch DN, Bank 15°

180° Point………..… Level Pitch, Bank 0°

Steep Spiral

Altitude………Sufficient for 3 360° turns

Clearing Turns………………….Complete

BCGUMPS……………………..Complete

Reference Point…………………….Select

Throttle……………………………….Idle

Airspeed……………...................76 KIAS

Spiral……………………...Max. Bank 60°

Entry………………Downwind ½ mi from

reference point

Radius…………...Maintain constant ½ mi

Engine…………...Clear once each turn on 			       the upwind

Ground Track………….Adjust for wind to 		maintain ½ mi radius

Chandelles

Altitude……………...Min Alt. 1500 AGL

Clearing Turns………………….Complete

BCGUMPS……………………..Complete

Airspeed ………………………...95 KIAS

Establish Bank ………………………. 30˚

Throttle……………………………… Full

0° - 90° Point

Throttle……………………..Full

Bank………………30° Constant

Pitch………...Gradually Increase

90° - 180° Point

Bank…………...Gradual Rollout

Pitch…………………...Constant

180° point

Airspeed……….5 kts above stall

Bank………………………….0°

Rectangular Course

Select Rectangular field for maneuver

Clearing Turns………………….Complete

BCGUMPS …………………….Complete

Throttle………………….2000-2200 RPM

Airspeed …………………..........90 KIAS

Altitude ………………...600-1000 ft AGL Entry…………….Downwind 45° ¼ -½ mi

away from reference area

Ground Track……….Adjust for wind drift  	    to maintain ¼ -½ mi distance

S -Turns Along a Road

Select road perpendicular to wind

Clearing Turns………………….Complete

BCGUMPS …………………….Complete

Throttle………………….2000-2200 RPM

Airspeed ………………….........100 KIAS

Altitude ………………...600-1000 ft AGL Entry…………….Downwind 45° ¼ -½ mi

away from reference area

Ground Track……….Adjust for wind drift  	    to maintain ¼ -½ mi distance

Turns Around a Point

Select a clearly defined point i.e. 4-Way Intersection

Clearing Turns………………….Complete

BCGUMPS …………………….Complete

Throttle………………….2000-2200 RPM

Airspeed …………………...........90 KIAS

Altitude ………………...600-1000 ft AGL Entry…………….Downwind 45° ¼ -½ mi

away from reference area

Ground Track……….Adjust for wind drift  	    to maintain ¼ -½ mi distance

Eights on Pylons

Clearing Turns………………….Complete

BCGUMPS……………………..Complete

Throttle…………………………100 KIAS

Reference Point(s)………………Selected

Entry………Downwind 45° at Pivotal Alt.

PA= (GSxKIAS)

11.3

Bank……………………………..30° - 40°

Piper PA28-181 Emergencies&Abnormals

Engine Fire-Start

Starter……………..Continue Cranking

Mixture ………….……....Idle Cut-Off

Throttle …………………………Open

Fuel Pump ………………………..Off

Fuel Selector ……………………..Off

Engine Failure-Takeoff

Airspeed …….Maintain Safe Airspeed

Land ………………….Straight Ahead

If Sufficient altitude has been gained attempt to restart:

Airspeed ……Maintain Safe Airspeed

Fuel Selector ………….Switch Tanks

Electric Fuel Pump ………………On

Mixture …………………..……Rich

Carburetor Heat …………………On

If power is not restored, proceed to POWER OFF LANDING

Engine Failure-In Flight

Airspeed …………………….76 KIAS

Best Landing Site……………...Locate

Fuel Pump …………………………On

Fuel Selector …………...Switch Tanks

Mixture ………………….………Rich

Carburetor Heat …………………...On

Engine Gauges ………………...Check

Primer …………………Check Locked

Ignition …………………..Check Both

If power is NOT restored proceed to POWER OFF LANDING:

Power Off Landing

Airspeed ………………………...76 KIAS

Best Landing Site …………………Locate

Passengers…………………………Prepare

If time and altitude permit:

Transponder ….................................. 7700

Radios …........................... Transmit 121.5

ELT ….................................................. On

When committed to landing:

Throttle …........................................... Idle

Mixture ….............................. Idle Cut-Off

Fuel Selector …................................... Off

Magnetos …......................................... Off

Master Switch …................................. Off

Belts/Shoulder Harness ….............. Secure

Door..………………………………..Open

Approach Speed …...................66-70 KIAS

Flaps……Extend (time/altitude permitting)

Engine Fire-Flight

Fuel Selector ………………………….Off

Mixture ……………………...Idle Cut-Off

Throttle ………………………………Idle

Fuel Pump …………………………….Off

Fresh Air Vents/Windows…………...Open

Panel Cabin Air/Heat/Defrost……...Closed

Magneto Switch ………………………Off

Master Switch ………………………...Off

If fire extinguishes, proceed to POWER OFF LANDING

If fire does not extinguish, proceed to EMERGENCY DESCENT

Electrical Fire

Batt/Alt Master Switch …......................Off

Radio Master and All Electrical……….Off

Fresh Air Vents/Windows…………...Open

Panel Cabin Air/Heat/Defrost……...Closed

If fire appears out and electrical power is necessary:

Batt/Alt Master Switch ….................... On

Circuit Breakers …........ DO NOT RESET

Radio/Electrical Equip---ON one at a time

*Land As Soon as Practical*

Emergency Descent

Throttle …............................................ Idle

Mixture …........................Rich (unless fire)

Bank …......................................... 30º – 45º

Airspeed………………………...125 KIAS

Alternator Failure (ALT Light)

Prepare for ELECTRICAL FIRE

All Non-Required Equipment………….Off

12V Charging Socket……………..Unplug

Circuit Breakers……………………Check

If no circuit breakers popped…

Alternator Master Switch……Cycle Off/On

If circuit breakers popped, reset them only if absolutely necessary for safety of flight.

If Alternator is not restored, land as soon as practical.

Engine Roughness

Throttle……………………….2500 RPM

Carburetor Heat ….............................. On

Fuel Pump…………………………….On

If roughness continues after 30 sec:

Fuel Selector …....................Switch Tanks

Check each tank for 30 sec

Mixture …...................................Full Rich

Engine Gauges …........................... Check

Magneto Switch ….......Check Individually

If operation is satisfactory on either magneto, continue on that magneto at reduced power and Full Rich mixture.  Land as soon as practical.

If roughness continues, land as soon as possible.

Loss of Oil Pressure (OIL) Light

*Land as Soon as Practical*

Prepare for POWER OFF LANDING

Loss of Fuel Pressure

Fuel Pump …………………………….On

Fuel Selector ………………..Fullest Tank

High Oil Temperature

Altitude……………….Level (stop climb)

Throttle………………………..2200 RPM

If temp does not go down, or continues to rise, land as soon as practical.

Prepare for POWER OFF LANDING

Open Door

Airspeed ……………Slow to 87 KIAS

Cabin Vents ……………………Close

Storm Window …………………Open

Door ………………Secure Top Latch

Spin Recovery

Throttle .......................................... Idle

Ailerons	................................... Neutral

Rudder…………………..Full opposite

(to the direction of rotation)

Yoke ….............................Full forward

When rotation stops…

Rudder …................................. Neutral

Yoke …...................Recover from dive

Throttle ….....Do not exceed 154 KIAS

IN AN EMERGENCY

Call 911 FIRST if necessary for safety

Then call Lake Elmo Aero at 651-777-1399